TurboFan_7

TurboFanLab / TurboModel User Manual

Browser Edition - Revision 1.13

Author: John Wojnaroski | Email: castle.64@centurylink.net

TurboFan_7 is a high-fidelity, software model of a modern turbofan engine intended for engineering development, simulation, and test.

# Contents

1. Purpose and Scope

2. System Architecture

3. TurboFanLab

3.1 Starting the Lab

3.2 Main controls

3.3 Test Conditions and Inlet Values

3.4 Starting an engine

3.5 Test Guide

3.6 LOCAL TEST and SIMULATOR modes

3.7 Selecting data for a test flight

3.8 NETWORK panel

3.9 Development workflow

3.10 Tips for Modeling

4. Working with Model Parameters

4.1 Parameter states

4.2 Saving and Reusing Parameter Files

4.3 TurboModel Update

5. TurboMgr Modes

5.1 Operating Modes

5.2 Control and Data Interface

6. Parameters

Appendix - Modeling Turbofans

# 1. Purpose and Scope

TurboFan_7 is a high-fidelity, software model of a modern turbofan engine intended for engineering development, simulation, and test.

TurboModel performs the engine calculation. Its behavior is produced from coupled thermodynamic, aerodynamic, mechanical, and control relationships so that airflow, pressure, temperature, fuel flow, spool speed, turbine/compressor work, thrust, and transient response evolve from the modeled engine physics rather than from a scripted sequence of indications.

TurboMgr provides the multi-engine control and data interface. It starts and manages the required TurboModel engine instances, routes commands and operating conditions to the correct engine, and returns calculated thrust and engine data to the host simulation. The managed architecture is intended to support as many as four independent engines.

TurboFanLab is the engineering interface used to develop and define an engine configuration, conduct controlled static and flight-coupled testing, adjust exposed model parameters, monitor internal response, and collect engineering data. During flight testing it provides the test and recording link used to capture model data and video for later analysis, comparison, and diagnosis of model behavior or problems.

The objective is a single engine model that can be developed and evaluated independently, then operated through TurboMgr in a simulation without creating a second or simplified version of the engine physics.

# 2. System Architecture

TurboMgr

Simulation-interface and routing layer. Receives simulation controls and environmental data, routes them to the selected engine, and returns thrust, engine state, and instrumentation.

TurboModel

Owns the engine physics, FCU, component maps, spool dynamics, gas path, thrust/reverse thrust, and model baseline parameters.

TurboFanLab

Engineering/test interface for model development, parameter adjustment, instrumentation, data/video capture, static testing, and flight-test support.

TurboMgr is the simulation-facing boundary for the engine package. It accepts the defined controls and operating environment from the host simulation, routes them to the appropriate TurboModel engine channel, and returns calculated thrust and engine data. TurboMgr is designed to support configurations of up to four independent engine instances.

# 3. TurboFanLab

TurboMgr and TurboModel provide the normal simulation engine path; TurboFanLab is added when engineering or test support is required.

TurboMgr starts and manages the required TurboModel engine instance or instances, establishes their control and data channels, routes the host-simulation commands and operating conditions, and returns the calculated engine results. TurboModel uses its accepted baseline values and performs the engine calculation independently.

TurboFanLab is not required for normal flight/simulation operations. When test or development support is requested, TurboMgr and the managed engine channels make the required engine data available to TurboFanLab so the selected engine can be monitored, adjusted, and recorded without changing the simulation-facing interface.

## What TurboFanLab adds

Live access to selected model parameters during controlled development tests.

Save/load of experimental parameter files for repeat testing and comparison.

Engineering instrumentation and synchronized data/video capture for analysis.

Static-test operation without a host flight simulation.

Flight-test support while the host simulation, TurboMgr, and TurboModel continue to use the normal control/data path.

TurboFanLab is the engineering console for starting and exercising a selected TurboModel engine, choosing test data, monitoring the model, and recording static or simulator-coupled tests.

## 3.1 Starting the Lab

1. Start the required TurboFan_7 operating configuration. For a static engine test, start the TurboModel/TurboFanLab configuration. For simulator-coupled testing, start the simulator/TurboMgr/TurboModel configuration and then TurboFanLab when engineering support or recording is required.

2. Open the Lab display. The standard TurboFanLab launcher starts the local web console on port 8080; open the address displayed by the launcher, normally the local TurboFanLab page.

3. Confirm the engine link. The status dot beside the parameter panel should turn green and the status line should report TurboModel shared-memory link active. This confirms TurboFanLab is communicating with the selected engine process; it does not by itself prove that an external simulator is connected.

4. Confirm the active parameter source. The PARAMS window at the top of the Lab identifies the active calibration source. DEFAULT means the accepted TurboModel baseline is in use. After LOAD, the window identifies the loaded parameter file; if a live value is then changed, the display indicates that the active calibration has been edited.

5. Choose the control source. Select LOCAL TEST for bench/static operation. Select SIMULATOR when the external simulation is to supply aircraft controls and flight/environment data through TurboMgr, including throttle, reverse, starter/start, fuel-cutoff state, altitude, speed/Mach, temperature, and pressure.

Actual TurboFanLab running screen captured during a start-to-idle test. The graphs are live, FCU and ENGINE logging are selected, the engine is at idle with the combustor burning, and the left NETWORK panel is in LOCAL TEST. The current build still uses legacy FG prefixes on two NETWORK readouts; those fields are simulator-supplied air/altitude and temperature/pressure data.

## 3.2 Main controls

| Control | What it does |
| --- | --- |
| Default | Restores the exposed model parameters to the accepted TurboModel baseline. |
| Load / Save | LOAD selects a saved calibration parameter file; SAVE writes the current adjustable calibration values to a named parameter file. |
| BUILD (planned) | Creates a new compiled TurboModel DEFAULT baseline from the currently selected saved parameter file. BUILD requires confirmation before the rebuild begins; the internal implementation is TBD. |
| Start | In LOCAL TEST, commands starter engagement and begins dry motoring with fuel cut off. In SIMULATOR mode, the simulator may supply the starter/start command through TurboMgr. |
| Fuel Cutoff | In LOCAL TEST, toggles CUTOFF / OPEN. In SIMULATOR mode, the simulator may supply the fuel-cutoff/run command through TurboMgr. |
| Reset Engine | Returns the selected development engine to the reset condition for a new bench/static test. |
| Pause / Resume Physics | Freezes or resumes the engine calculation and live graphs during engineering work. |
| F8 RECORD | Starts/stops the matched timestamped test capture. One CSV is created for each selected DATA LOG group; the capture package can also include the synchronized visual recording. |
| TEST | Names the run. The selected Test Guide preset can write its test name into this field. |
| PARAMS | Shows DEFAULT, the loaded parameter file, or an edited state so the calibration used for the run can be identified. |
| Test Guide | Opens the built-in FCU timing/test guide. It displays the setup, sequence/timing, and watch items for the selected test; Use This Test copies the preset name into TEST but does not execute the test automatically. |

## 3.3 Test Conditions and Inlet Values

Test Conditions. In LOCAL TEST these controls establish the external boundary conditions used by the engine model. In SIMULATOR mode the corresponding flight/environment values are supplied by the simulator through TurboMgr and the Lab becomes primarily an observation/engineering display for those conditions.

| Readout | Meaning |
| --- | --- |
| Altitude (ISA set) | Geometric/test altitude in feet. In the current Lab test controls it is constrained to the supported test envelope; simulator-coupled operation receives altitude from the host. |
| Airspeed | True airspeed command/display in knots used with atmospheric conditions to establish the inlet flight state. |
| Ambient temp | Static ambient temperature in °F at the commanded/test condition. |
| Ambient pressure | Static ambient pressure in inHg at the commanded/test condition. |

### Inlet Values — derived engine-entry and compressor quantities

The Inlet Values panel is read-only. It shows the conditions TurboModel is actually using after the atmospheric/flight inputs have been converted to inlet total conditions and corrected quantities. These values are especially useful when comparing the same engine calibration at different altitude, temperature, pressure, or airspeed conditions.

| Readout | Meaning |
| --- | --- |
| Mach | Calculated flight Mach number for the current operating condition. |
| Density alt | Atmospheric density expressed as an equivalent standard-atmosphere altitude. It can be negative on a cold/high-pressure day even though commanded geometric altitude is not below sea level. |
| Tt2 | Total temperature at the engine inlet/fan face after the flight-condition ram-temperature effect is applied. |
| Pt2 | Total pressure at the engine inlet/fan face after flight-condition ram pressure and inlet pressure recovery/losses are applied. |
| N1c | Low-spool speed corrected to the inlet reference condition for component-map use and comparison. |
| N2c | High-spool speed corrected to the inlet reference condition for component-map use and comparison. |
| Wc inlet | Corrected total inlet airflow in lb/s. |
| Wc core | Corrected core airflow in lb/s used with the core/compressor map solution. |
| W core | Actual current core mass flow in lb/s. |
| HPC PR | Current high-pressure-compressor total pressure ratio at its solved map operating point. |
| HPC η | Current HPC efficiency at the solved operating point. |
| HPC work | Current shaft power absorbed by the HPC, shown in kW; it is part of the HP-spool work balance against HPT output and other loads. |

Why these numbers matter. N1c, N2c, corrected flows, HPC pressure ratio, efficiency, and work make it possible to determine whether a change in engine response comes from the atmospheric boundary condition, the component-map operating point, or the control system rather than from the raw N1/N2 indications alone.

## 3.4 Starting an engine

In LOCAL TEST the Lab start controls act on the engine channel currently assigned to TurboFanLab. In SIMULATOR mode, starter and fuel-cutoff commands can come from the simulator through TurboMgr along with throttle, reverse, and environment data. The NETWORK panel identifies the active engine/channel and the source traffic.

1. Set FUEL CUTOFF to CUTOFF and, if necessary, press Reset Engine before beginning a new start test.

2. Press Start. The starter flag changes to STARTER ENGAGED and N2 begins to rise through the normal rotor-inertia calculation.

3. At the configured lightoff N2, press FUEL CUTOFF once to OPEN. With the current baseline values this occurs near 22% N2; if a different parameter file is loaded, use the active lightoff setting.

4. Verify lightoff. COMBUSTOR BURNING should appear, EGT and fuel flow should rise, and N2 should continue accelerating toward idle.

5. Verify starter cutout and idle stabilization. The starter disengages at the configured cutout point (55% N2 in the baseline shown here), after which the engine should continue under turbine power and settle near the current idle target.

6. For shutdown, return FUEL CUTOFF to CUTOFF. Use Reset Engine only when a reset condition is required for the next test.

## 3.5 Test Guide

The Test Guide button opens the built-in FCU Timing Test Guide. It is a procedure reference, not an automatic test sequencer. The operator remains in control of the engine and of F8 recording.

| Readout | Meaning |
| --- | --- |
| Test selector | Selects one of the current predefined tests (T01 through T11 in this build), such as Start to Idle, Idle Stability, acceleration/deceleration checks, map tests, reverse geometry, or the FCU diagnostic sequence. |
| Setup | Shows the required initial engine state, ambient/test conditions, and any parameter/logging constraints. |
| Sequence / timing | Shows the control sequence, stabilization periods, and when to start/stop F8 recording. |
| Watch | Lists the engine variables, transients, peaks, delays, limiter behavior, or other effects that should be examined in the recorded data. |
| Use This Test | Copies the selected preset name into the TEST field and synchronizes that name for the recording package. It does not move the throttle, start the engine, change fuel cutoff, or start F8 recording. |
| Close | Closes the guide without running anything. |

## 3.6 LOCAL TEST and SIMULATOR modes

| Mode | Control authority | Typical use |
| --- | --- | --- |
| LOCAL TEST | TurboFanLab owns the forward throttle and reverse controls. Start, fuel cutoff, test conditions, parameters, monitoring, and recording are available locally. | Static tests, starts, map work, FCU tuning, reverse testing, and troubleshooting without a flight simulator. |
| SIMULATOR | The external simulator supplies forward throttle, reverse, starter/start, fuel-cutoff state, and the flight/environment inputs carried through TurboMgr. TurboFanLab remains available for monitoring, parameter work, test naming/guidance, data-log selection, and recording. | Flight-test work in which the engine follows the aircraft controls and atmosphere while TurboFanLab records and displays the same engine in real time. |

Simulator data on the Lab screen. When SIMULATOR is selected and valid packets are arriving, the NETWORK panel shows the received control values and the simulator-supplied aircraft/environment quantities. The Test Conditions/Inlet Values areas then provide the corresponding operating-condition view used to interpret the engine response.

## 3.7 Selecting data for a test flight

Choose the DATA LOG groups before pressing F8. TurboFanLab writes one timestamped CSV for each selected group; the F8 capture system also provides the matched visual recording used for later analysis.

Recommended normal flight-test selection. Captures the fuel/control response and the principal engine state needed to evaluate N1/N2, EGT, fuel flow, thrust, commands, and transients.

Add only when the test is aimed at component-map behavior or reverser operation. Keeping the selection focused makes later analysis easier.

Use for short diagnostic runs when a broad capture is specifically needed. It produces substantially more data than a focused test.

The NETWORK group is present in the current Lab layout but may be disabled until network telemetry logging is enabled in the running build. When available, it is useful for interface timing or simulator-link diagnostics.

1. Enter a meaningful TEST name before the run.

2. Select FCU and ENGINE for a normal test flight; add only the specialized groups required by the test objective.

3. Press F8 before the event you want captured and confirm the banner changes to RECORDING.

4. Run the planned start, taxi, takeoff, climb, cruise, transient, reverse, or shutdown sequence.

5. Continue long enough after the final event to establish the post-event steady condition, then press F8 again to close the recording package.

## 3.8 NETWORK panel

| Field | Meaning |
| --- | --- |
| Control | LOCAL TEST means Lab control. SIMULATOR means the external simulation supplies throttle, reverse, start/starter, fuel-cutoff state, and flight/environment inputs through TurboMgr. |
| Status | LINK ACTIVE means the network sockets are ready and valid simulator control packets are being received. WAITING FOR SIM means the network endpoints are ready but valid control traffic has not yet arrived. UDP ERROR means the network endpoints were not successfully established. |
| Engine | The engine/channel whose simulator traffic and engine return data are being displayed. |
| Receive | The local address and port accepting simulator control/environment packets. An address of 0.0.0.0 means the receiver is bound to the available local network interfaces. |
| Send | The simulator destination address and port used for returned engine data. 127.0.0.1 indicates the simulator is on the same computer; a LAN address indicates a separate simulator computer. |
| RX / TX | Counts of received and transmitted packets. Both should continue increasing during a healthy two-way connection. |
| RX ctl | The throttle (T) and reverse (R) commands most recently received from the simulator. Changing the simulator controls should be reflected here. |
| SIM AIR / ALT | Latest simulator-supplied true airspeed and altitude. The current screen capture may still show the legacy FG prefix. |
| SIM TEMP / P | Latest simulator-supplied static temperature and pressure. The current screen capture may still show the legacy FG prefix. |
| TX eng | The engine thrust and fuel-flow values currently being returned toward the simulator. |

Connection check. A green TurboModel shared-memory link active indication confirms the Lab-to-engine engineering connection. LINK ACTIVE in the NETWORK panel confirms valid external simulator traffic. In a flight-test configuration, both indications should be healthy, the RX/TX counters should advance, and the received aircraft/environment values should follow the simulator.

## 3.9 Development workflow

A typical development cycle starts with the default settings or an accepted model baseline, change one or a small group of parameters, run repeatable test, record the response, save promising settings, and finally adopt an accepted calibration into the next model baseline.

1. DEFAULT — restore exposed parameters to the current TurboModel baseline.

2. Adjust — change the parameter or parameter group being evaluated.

3. Test — run a controlled start, steady-state point, transient, reverse sequence, static test, or simulator-coupled test.

4. Record — capture the required engineering data and, when useful, synchronized video for later analysis.

5. SAVE — preserve a promising calibration as a named parameter file.

6. LOAD — restore a previously saved parameter file for repeat testing or comparison.

7. BUILD — after validation, use the selected saved parameter file to create the next compiled TurboModel DEFAULT baseline. BUILD is a planned TurboFanLab function; implementation is TBD.

## 3.10 Tips for Modeling

Change as few parameters as possible per run so the cause of a response remains identifiable.

Use consistent test points and stabilization times when comparing calibrations.

Record enough data to explain the result, not merely to show that a value changed.

Save a named parameter file before moving to the next calibration branch.

Treat maps, spool dynamics, fuel/control gains, combustion, gas-path geometry, and reverse calibration as distinct physical areas; avoid compensating for a deficiency in one subsystem by distorting another.

After accepting a calibration, repeat the relevant regression tests from the updated baseline.

# 4. Working with Model Parameters

TurboFanLab presents selected TurboModel parameters as an engineering control panel. Parameters are organized by primary topic in the same general form used by the Lab. Select a topic to see its parameter names; select an individual parameter to see its description, baseline value, allowed range, and adjustment step.

Changing an exposed parameter changes that value in the running development engine, allowing the effect to be observed immediately in a controlled test. The normal simulation interface does not change when a Lab parameter is adjusted.

## 4.1 Parameter states

| State | Meaning | Use |
| --- | --- | --- |
| Model baseline | The accepted parameter values used when TurboModel starts. | Normal simulation operation and the starting point for development. |
| Live Lab settings | Temporary values being evaluated in the selected development engine. | Development, comparison, troubleshooting, and tuning. |
| Saved parameter file | A named file containing Lab calibration settings saved for later reuse. | Repeat tests, compare calibrations, and preserve promising setups. |

## 4.2 Saving and Reusing Parameter Files

When TurboFanLab starts, it reads the current TurboModel baseline values and presents those values as the DEFAULT parameters for the engineering session. That compiled baseline remains the reference point until a different calibration is deliberately loaded for testing.

SAVE writes the current adjustable calibration values to a named parameter file. The file contains calibration settings only; it does not contain the instantaneous running condition of the engine such as N1, N2, EGT, spool acceleration, actuator position, or elapsed test time.

LOAD begins with the current compiled baseline and then applies the calibration values contained in the selected parameter file to the running development engine. LOAD does not alter the TurboModel source or compiled DEFAULT baseline.

DEFAULT ¦ LOAD ¦ SAVE ¦ BUILD. DEFAULT, LOAD, and SAVE support calibration development. BUILD is the planned action that promotes one named saved parameter file into the next compiled TurboModel DEFAULT baseline. See Section 4.3.

## 4.3 TurboModel Update

The TurboFanLab parameter menu is DEFAULT | LOAD | SAVE | BUILD. LOAD and SAVE are used to develop and preserve calibrations. BUILD is the planned TurboFanLab function that takes one selected saved parameter file and makes that calibration the compiled DEFAULT baseline of a new TurboModel build.

1. LOAD the intended parameter file. In TurboFanLab, select LOAD and choose the saved parameter file that is to become the new baseline. Confirm that the PARAMS indication identifies the intended file.

2. Test and verify it. Run the required static or simulator-coupled checks with that file loaded. The file selected for BUILD should be the calibration that has completed the required development and verification work.

3. SAVE any final changes. If parameters are changed after LOAD, save those changes to the intended named parameter file, as BUILD operates on a saved file, not on unidentified or unsaved live edits.

4. Press BUILD. TurboFanLab opens a confirmation window identifying the exact saved parameter file that will be used for the new TurboModel DEFAULT baseline.

One compiled DEFAULT per build. Rev 1.13 treats each TurboModel executable as having one compiled DEFAULT baseline. Other saved parameter files may be retained and loaded for development or comparison, but they do not become the compiled DEFAULT until that file is selected and used in a BUILD operation.

# 5. TurboMgr Modes

TurboMgr owns the simulation-facing engine interface and manages the TurboModel engine processes. The engine applications use the same control/data relationships whether the host simulation is on the same computer or reached over a LAN.

When everything runs on one computer, the simulation and TurboMgr can communicate through the local loopback interface (127.0.0.1). For a separate engine computer, TurboMgr uses the configured LAN addresses. Network configuration is established as part of the TurboFan_7 setup; once configured, the location of the connection is intended to be transparent to TurboModel and to normal engine operation.

## 5.1 Operating Modes

Flight Mode

Simulation + TurboMgr + TurboModel

Normal simulation operation. TurboMgr starts and manages the required engine instances, receives controls and flight/environment data from the simulation, and returns thrust and engine data. TurboFanLab is not required.

Static Test Mode

TurboMgr + TurboModel + TurboFanLab

Engine development without a flight simulation. TurboFanLab supplies the engineering/test interface for commands, conditions, parameter adjustment, monitoring, and recording while TurboMgr manages the selected engine process.

Flight Test Mode

Simulation + TurboMgr + TurboModel + TurboFanLab

The normal simulation control/data path remains active while TurboFanLab provides engineering observation and test recording for the selected engine. This mode supports collection of flight-test data and video for later analysis and diagnosis.

## 5.2 Control and Data Interface

| Direction | Typical information |
| --- | --- |
| Simulation → TurboMgr | Throttle/thrust-lever commands, start and fuel-control commands, reverse command, engine selection/channel information, and required flight/environment data such as altitude, Mach/airspeed, pressure, and temperature. |
| TurboMgr → TurboModel | Per-engine commands and operating conditions routed to the appropriate managed engine instance. |
| TurboModel → TurboMgr | Calculated thrust and engine-state/instrumentation data such as N1, N2, EGT, fuel flow, and other channels defined by the interface. |
| TurboMgr → Simulation | Per-engine thrust and engine data returned to the host simulation for dynamics, indications, systems, and recording. |

Multi-engine operation. TurboMgr is designed around independent engine channels and can manage configurations of up to four TurboModel engines. The host simulation sees the defined engine interface rather than the internal engine equations.

# 6. Parameters

Select a primary topic in either column to display its parameter names, then select a parameter to open its description, baseline value, adjustment range, and step size. Use the search box at the top of the manual to locate a parameter directly.

Parameter topics are closed by default; open only the topic or parameter you want to inspect.

Start / Control

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Idle throttle detent | Forward-throttle idle stop; in local operation the throttle command cannot fall below this value while reverse is stowed. | 0.6 | 0.5 to 0.7 | 0.01 |
| Fuel lightoff N2 | N2 speed at which the FCU permits fuel for lightoff during the start sequence. | 22 % | 15 to 30 % | 0.5 % |
| Starter cutout N2 | N2 threshold used to remove starter drive after the engine has accelerated through the start sequence. | 55 % | 35 to 55 % | 0.5 % |
| Idle N2 target | Gas-generator N2 target commanded by the FCU at the idle throttle detent. | 60 % | 50 to 70 % | 0.5 % |
| Full throttle N2 target | Gas-generator N2 target at full forward throttle; the forward throttle schedule interpolates toward this value. | 100 % | 90 to 104 % | 0.25 % |
| N2 starter power | Mechanical starter power available to the N2 spool during engine start. | 300 kW | 50 to 300 kW | 5 kW |
| N2 starter torque limit | Maximum starter torque that may be applied to the N2 spool; caps starter torque even when the power command would imply more. | 550 N m | 100 to 1,200 N m | 10 N m |

Fuel / N2 PID

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Idle feed-forward FAR | Feed-forward fuel/air-ratio anchor used near idle before PID correction is added. | 0.0148 F/A | 0.001 to 0.03 F/A | 0.0001 F/A |
| Intermediate feed-forward N2 | N2 location of the intermediate feed-forward anchor in the low-to-high power fuel schedule. | 70 % | 61 to 99 % | 0.5 % |
| Intermediate feed-forward FAR | Feed-forward FAR assigned to the intermediate N2 anchor; shapes the schedule between idle and full power. | 0.01545 F/A | 0.001 to 0.04 F/A | 0.0001 F/A |
| Full-power feed-forward FAR | Feed-forward FAR anchor used at the full-power end of the forward-throttle schedule. | 0.0247 F/A | 0.005 to 0.04 F/A | 0.0001 F/A |
| Lightoff FAR boost | Additional FAR applied at lightoff to support start combustion; it fades as N2 approaches the idle target. | 0.0032 F/A | 0 to 0.03 F/A | 0.0001 F/A |
| Maximum FAR | Absolute upper clamp on commanded fuel/air ratio. | 0.04 F/A | 0.01 to 0.06 F/A | 0.0001 F/A |
| N2 PID Kp | Proportional N2 control gain that converts instantaneous N2 error into a FAR correction. | 0.00028 FAR/% | 0 to 0.0015 FAR/% | 5e-06 FAR/% |
| N2 PID Ki | Integral N2 control gain that accumulates persistent N2 error into a FAR correction over time. | 4e-05 FAR/%/s | 0 to 0.0002 FAR/%/s | 1e-06 FAR/%/s |
| N2 PID Kd | Derivative-on-measurement N2 gain that adds damping from measured N2 rate of change. | 3e-05 FAR s/% | 0 to 0.0003 FAR s/% | 5e-06 FAR s/% |
| N2 derivative filter | Time constant for the filtered N2-rate signal used by the derivative term; larger values provide more smoothing. | 0.15 s | 0 to 1 s | 0.01 s |
| Integral minimum | Lower bound on the N2 PID integral contribution in FAR units. | -0.02 FAR | -0.04 to 0 FAR | 0.0005 FAR |
| Integral maximum | Upper bound on the N2 PID integral contribution in FAR units. | 0.02 FAR | 0 to 0.04 FAR | 0.0005 FAR |
| Anti-windup gain | Back-calculation anti-windup gain that drives the integrator toward a deliverable FAR when another limiter is active. | 4 1/s | 0 to 20 1/s | 0.25 1/s |

Fuel / Limits

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| N2 limiting begins | N2 value at which overspeed fuel limiting begins. | 102 % | 98 to 104 % | 0.25 % |
| N2 absolute maximum | Upper N2 endpoint of the overspeed protection band; used with the overspeed fuel-reduction schedule. | 104 % | 100 to 108 % | 0.25 % |
| N2 overspeed fuel reduction | Amount of FAR removed per percent N2 in the overspeed limiting region. | 0.004 FAR/% | 0 to 0.02 FAR/% | 0.0005 FAR/% |
| EGT limiting begins | EGT at which temperature-based fuel limiting begins. | 900 C | 650 to 1,000 C | 5 C |
| Maximum EGT | Upper EGT endpoint of the temperature protection band. | 950 C | 750 to 1,100 C | 5 C |
| Fuel increase slew | Maximum rate at which metered fuel flow may increase; limits acceleration-side fuel slew. | 0.2 kg/s2 | 0.02 to 1 kg/s2 | 0.01 kg/s2 |
| Fuel decrease slew | Maximum rate at which metered fuel flow may decrease; limits deceleration-side fuel slew. | 0.5 kg/s2 | 0.02 to 1.5 kg/s2 | 0.01 kg/s2 |

Fan / Compressor

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| N1 100% design speed | Physical shaft speed represented by 100 percent N1, used to convert between N1 percentage and LP-spool rotational speed. | 5,200 rpm | 3,500 to 7,000 rpm | 25 rpm |
| N2 100% design speed | Physical shaft speed represented by 100 percent N2, used to convert between N2 percentage and HP-spool rotational speed. | 14,500 rpm | 9,000 to 18,000 rpm | 50 rpm |
| Design core mass flow | Core mass-flow design anchor used to scale the generic compressor-map solution to the simulated engine size. | 58 kg/s | 20 to 100 kg/s | 0.5 kg/s |
| Bypass ratio | Design bypass-to-core mass-flow ratio; sets the relationship between fan/bypass flow and core flow. | 5.1 | 2 to 8 | 0.05 |
| Fan design pressure ratio | Fan total-pressure-ratio design anchor used in fan map scaling. | 1.55 Pt out/in | 1.05 to 2.2 Pt out/in | 0.01 Pt out/in |
| LPC design pressure ratio | LPC/booster total-pressure-ratio design anchor used in LPC map scaling. | 1.75 Pt out/in | 1.05 to 3 Pt out/in | 0.01 Pt out/in |
| Fan/LPC PR speed exponent | Selected LPC/booster map Rline operating coordinate. | 3 | 1 to 3 | 0.05 |

Fan Map Step 3

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Fan design efficiency (map anchor) | Speed-shaping exponent retained for the fan/LPC pressure-ratio schedule around the mapped design anchors. | 0.88 | 0.5 to 4 | 0.05 |
| Fan map Rline | Fan design efficiency anchor used to scale the generic fan map efficiency to the simulated engine. | 2.2 | 0.5 to 0.98 | 0.005 |

LPC Map Step 4

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| LPC design efficiency (map anchor) | Selected fan-map Rline operating coordinate used by the staged map solution. | 0.86 | 1 to 3 | 0.05 |
| LPC map Rline | LPC/booster design efficiency anchor used to scale the generic LPC map. | 2.15 | 0.5 to 0.98 | 0.005 |

HPC Map Step 1

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| HPC design pressure ratio | HPC design-point pressure-ratio scaling anchor, Pt3/Pt25, applied to the generic HPC map. | 11 Pt3/Pt25 | 5 to 20 Pt3/Pt25 | 0.05 Pt3/Pt25 |
| HPC design efficiency | HPC design-point efficiency scaling anchor applied to the generic HPC map. | 0.86 | 0.6 to 0.95 | 0.0025 |
| HPC map Rline | Selected HPC map Rline operating coordinate used while the staged model does not solve Rline as a full nonlinear cycle variable. | 2.05 | 1 to 3 | 0.05 |

Spool Dynamics

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| N1 spool inertia | Rotational inertia of the LP spool (fan + booster/LPC + LPT); higher inertia slows N1 acceleration and deceleration for a given net torque. | 55 kg m2 | 20 to 250 kg m2 | 1 kg m2 |
| N2 spool inertia | Rotational inertia of the HP spool (HPC + HPT); higher inertia slows N2 acceleration and deceleration for a given net torque. | 12 kg m2 | 2 to 60 kg m2 | 0.5 kg m2 |
| N1 friction coefficient | LP-spool friction coefficient used in the shaft-load model. | 0.06 | 0 to 0.2 | 0.005 |
| N2 friction coefficient | HP-spool friction coefficient used in the shaft-load model. | 0.02 | 0 to 0.12 | 0.002 |
| N2 accessory load | Mechanical accessory load removed from the N2/HP spool. | 10 kW | 0 to 100 kW | 1 kW |
| Physics time step | Engine physics integration time step used by the dynamic model. | 0.005 s | 0.001 to 0.02 s | 0.001 s |

Turbine

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Shaft mechanical efficiency | Mechanical efficiency applied when converting turbine gas-path work into usable shaft work. | 0.98 | 0.8 to 1 | 0.005 |
| HPT map design efficiency | HPT map design-efficiency anchor for the turbine that supplies work to the N2/HP spool. | 0.9 | 0.5 to 0.98 | 0.005 |
| LPT design efficiency (map anchor) | LPT map design-efficiency anchor for the turbine that supplies work to the N1/LP spool. | 0.91 | 0.5 to 0.98 | 0.005 |
| HPT design expansion ratio | HPT design total-pressure expansion ratio used to scale the HPT map/expansion solution. | 3.8 Pt in/out | 1.1 to 8 Pt in/out | 0.05 Pt in/out |
| LPT design expansion ratio | LPT design total-pressure expansion ratio used to scale the LPT map/expansion solution. | 3.2 Pt in/out | 1.1 to 8 Pt in/out | 0.05 Pt in/out |
| Design T4 | Design combustor-exit / turbine-inlet total temperature T4 used as a turbine scaling reference. | 1,550 K | 1,000 to 1,900 K | 10 K |
| HPT low-speed expansion floor | Minimum low-speed contribution in the HPT off-design expansion schedule. | 0.05 fraction | 0.01 to 0.5 fraction | 0.01 fraction |
| HPT expansion speed slope | Speed-dependent multiplier coefficient in the HPT off-design expansion schedule. | 0.95 | 0.2 to 1.5 | 0.05 |
| HPT expansion speed exponent | Exponent shaping how HPT expansion approaches the design condition as corrected speed rises. | 2 | 0.5 to 3.5 | 0.1 |
| LPT low-speed expansion floor | Minimum low-speed contribution in the LPT off-design expansion schedule. | 0.01 fraction | 0 to 0.3 fraction | 0.01 fraction |
| LPT expansion speed slope | Speed-dependent multiplier coefficient in the LPT off-design expansion schedule. | 0.99 | 0.2 to 1.5 | 0.05 |
| LPT expansion speed exponent | Exponent shaping how LPT expansion approaches the design condition as corrected speed rises. | 3 | 0.5 to 3.5 | 0.1 |
| Low-speed torque omega floor | Minimum shaft-speed fraction used when converting turbine power to torque at very low rpm, avoiding unrealistically large torque as omega approaches zero. | 0.12 N/Ndesign | 0.03 to 0.3 N/Ndesign | 0.01 N/Ndesign |
| Core nozzle pressure margin | Pressure margin above ambient used in the core-nozzle pressure/exhaust solution. | 1.03 Pa/Pamb | 1 to 1.25 Pa/Pamb | 0.005 Pa/Pamb |

Gas Path / Exhaust

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Inlet pressure recovery | Total-pressure recovery through the inlet before subsequent duct losses are applied. | 0.999 | 0.8 to 1 | 0.001 |
| Inlet duct loss | Fractional inlet-duct total-pressure loss applied to the incoming flow. | 0.001 fraction | 0 to 0.1 fraction | 0.001 fraction |
| Fuel heating value | Fuel lower-heating-value style energy input used by the combustor energy balance. | 43e6 J/kg | 35e6 to 50e6 J/kg | 100,000 J/kg |
| Combustion efficiency | Fraction of fuel chemical energy transferred into the modeled combustion gas stream. | 0.98 | 0.75 to 1 | 0.005 |
| Combustor pressure loss | Fractional total-pressure loss through the combustor. | 0.05 fraction | 0 to 0.15 fraction | 0.005 fraction |
| Nozzle efficiency | Efficiency used in converting available exhaust pressure/enthalpy into jet velocity and thrust. | 0.95 | 0.7 to 1 | 0.005 |
| Cascade axial turning | Physical cascade axial-turning term used in the raw reverse fan-momentum estimate, not the retired global reverse-thrust multiplier. | 0.75 | 0.3 to 1 | 0.01 |

Reverser Hardware

| Parameter | Description | Default | Range | Step |
| --- | --- | --- | --- | --- |
| Cascade/sleeve travel time | Time for the translating cascade/sleeve portion of the reverser to travel between commanded positions. | 0.75 s | 0.2 to 3 s | 0.05 s |
| Blocker-door travel time | Time for the blocker-door mechanism to travel between commanded positions. | 0.75 s | 0.2 to 3 s | 0.05 s |
| Raw fan reverse reference | Reference raw fan reverse-thrust magnitude used to normalize the reverse-geometry calibration. | 12,375 lbf | 2,000 to 25,000 lbf | 25 lbf |
| Maximum reverse fan thrust | Upper fan reverse-thrust magnitude produced by the calibrated reverse-geometry mapping. | 20,350 lbf | 5,000 to 30,000 lbf | 50 lbf |
| Reverse geometry shape | Shape parameter controlling the nonlinear mapping from raw fan reverse momentum to calibrated fan reverse thrust. | 9 | 0.5 to 20 | 0.1 |
| Reverse thrust transition time | Smoothing time used when transitioning reverse thrust to avoid an instantaneous force discontinuity. | 2 s | 0.25 to 5 s | 0.05 s |
| Detent 1 N2 command | N2 command associated with reverse Detent 1, with the FCU responsible for achieving the requested N2. | 70 % | 60 to 100 % | 0.5 % |
| Detent 2 N2 command | N2 command associated with reverse Detent 2. | 85 % | 60 to 100 % | 0.5 % |
| Maximum reverse N2 command | N2 command associated with maximum reverse. | 100 % | 60 to 104 % | 0.5 % |
| Stow allowed at/below N2 | Maximum actual N2 at which the reverser is allowed to begin stowing; higher N2 holds the system in idle reverse until spool-down. | 65 % | 55 to 75 % | 0.5 % |

# Appendix

## Modeling Turbofans

Adapting TurboModel to another engine family should preserve the governing physics and replace only the engine-specific architecture, geometry, component data, maps, limits, and control calibration required by the target engine.

Modeling integrity. Engine development should adhere rigorously to the underlying thermodynamic, aerodynamic, mechanical, and control science. Actual engine information and credible published or test data should be used wherever available; engineering estimates should be identified as estimates and replaced when better data becomes available. Calibration should refine the physical model, not conceal an error in one subsystem by forcing another subsystem to compensate for it.

What stays the same

Conservation of mass and energy, compressible-flow relationships, Brayton-cycle thermodynamics, corrected flow and corrected speed, compressor and turbine map methods, shaft work balance, rotational dynamics, nozzle momentum, and numerical time integration remain the foundation of the model.

What is engine-specific

Fan and core size, mass-flow capacity, bypass ratio, pressure-ratio split, component efficiencies, design shaft speeds, map scaling, turbine work, nozzle areas, spool inertia, accessory loads, combustor behavior, start schedule, fuel-control schedule, and reverse-thrust calibration must be matched to the target engine.

What may require a model-structure extension

The spool arrangement must match the engine. A two-spool target can use the existing two-spool structure as a starting point. Adding a third spool is primarily a software/model-structure extension within TurboModel: another independently integrated shaft is added with its compressor/turbine pair, work balance, inertia, speed, maps, limits, controls, and instrumentation. The governing aerodynamic, thermodynamic, and shaft-dynamic principles do not change.

### Suggested conversion workflow

1. Define the engine architecture. Identify the number of spools, fan/booster/compressor arrangement, turbine arrangement, bypass stream, mixer/nozzle configuration, and reverse-thrust arrangement.

2. Establish physical scale. Enter or derive fan diameter, representative flow areas, design mass flow, bypass ratio, core size, nozzle areas, and other dimensions that set the engine's flow capacity.

3. Establish the design point. Match rated thrust, design N1/N2 (and N3 where applicable), overall and component pressure ratios, corrected flows, turbine inlet conditions, efficiencies, and exhaust conditions at a known operating point.

4. Install or scale component maps. Use suitable fan, booster/LPC, HPC, and turbine maps; scale corrected flow, pressure ratio, efficiency, and speed coordinates so the maps reproduce the chosen design point while preserving useful off-design behavior.

5. Match shaft dynamics. Set spool inertias and accessory loads, then balance compressor power against turbine power on each shaft. Validate acceleration, deceleration, start, and wind-down response as well as steady thrust.

6. Calibrate fuel and limits. Tune lightoff, idle, acceleration/deceleration schedules, governing gains, temperature limits, fuel-air-ratio limits, and rated-speed commands for the target engine.

7. Validate the complete envelope. Compare sea-level static, altitude/Mach points, idle, intermediate and maximum power, transients, starts, and reverse operation against credible reference data before adopting the calibration.

### Examples

GE90 family

A two-spool GE90-type engine can use the existing two-spool model structure as a starting point. The physical scale, mass-flow capacity, bypass ratio, compressor and turbine characteristics, shaft speeds and inertias, nozzle geometry, control calibration, and thrust targets must be matched to the selected GE90 variant.

PW4000 family

A PW4000-type engine is also fundamentally compatible with a two-spool arrangement, but its geometry, flow capacity, maps, pressure-ratio distribution, spool dynamics, fuel schedule, limits, and performance targets must be represented independently rather than by simple thrust scaling.

Trent 500 family

The Trent 500 uses a three-spool arrangement. The same aerodynamic and thermodynamic principles still apply, but TurboModel must add the intermediate-pressure spool as its own dynamic shaft with the corresponding compressor and turbine work balance, maps, inertia, speed, limits, controls, and instrumentation. This third spool is principally a software extension of the existing model. TurboFanLab would provide a dedicated N3/IP-spool parameter group—potentially through a popup or separate parameter panel—so the additional spool can be configured and tuned without crowding the normal two-spool controls.

Modeling principle: adapting TurboModel to another engine family should preserve the physics and replace the engine-specific scale, component behavior, architecture, and calibration. A different engine is not created by multiplying thrust by a scale factor.

TurboFan_7 TurboFanLab / TurboModel User Manual · Browser Edition Rev 1.13 · John Wojnaroski · castle.64@centurylink.net